| Frequency Band | Left connector: 0.05 – 960 MHz |
| --- | --- |
|  | Right connector: 240 – 4000 MHz |
| Frequency Span (single sweep) | 2 – 960 MHz |
| Internal LNA Gain | Selectable, 25 dB |
| Internal Attenuators | Selectable, 30 dB and 60 dB |
| WiFi Analyzer | Supports 2.4 GHz band |
| Display | Graphics LCD, 128 × 64 pixels, great outdoor visibility |
| Backlight | Yes, for indoor visibility |
| Battery | Internal Lithium-Ion, 1800 mAh, rechargeable |
| Connectors | Standard SMA, 50 ohms |
| Included Antennas | Wideband 144/433 MHz dual-band telescopic antenna |
|  | UHF 400-900 MHz rubber duck articulated antenna |
|  | 2.4 GHz whip antenna |
| Amplitude Resolution | 0.5 dBm |
| Dynamic Range | -125 dBm to +10 dBm |
| Absolute Max Input Power | +30 dBm |
| Average Noise Level (typical LNA) | -125 dBm |
| Frequency Stability & Accuracy (typical) | +-1.0ppm |
| Amplitude Stability & Accuracy (typical) | +-3 dBm |
| Frequency Resolution | 1 kHz |
| Resolution Bandwidth (RBW) | Automatic, 2.6 kHz to 600 kHz |
| OS Support | Windows, Linux, MacOS X |